State code 5: Development in a state-controlled tunnel environment

Table 5.1 Development in a state-controlled tunnel environment

| Performance outcomes | Acceptable outcomes | Response |
| --- | --- | --- |
| Buildings and structures |  |  |
| PO1 The location of buildings, structures, infrastructure, services and utilities does not cause damage to a state-controlled transport tunnel, or obstruct state-controlled transport tunnel infrastructure. | AO1.1 Buildings, structures, infrastructure, services and utilities are not located on land identified as a state-controlled transport tunnel. AND AO1.2 Buildings, structures, infrastructure, services and utilities can be maintained without requiring access to land identified as a state-controlled transport tunnel. | Complies with PO# / AO# Use this column to indicate whether compliance is achieved with the relevant PO or AO (or if they do not apply), and explain why |
| PO2 Buildings, structures, infrastructure, services and utilities do not interfere with, or result in damage to, infrastructure or services in a state-controlled transport tunnel. | No acceptable outcome is prescribed. |  |
| PO3 Buildings, structures, infrastructure, services and utilities do not add or remove loading that will cause damage to a state-controlled transport tunnel or state-controlled tunnel infrastructure. | No acceptable outcome is prescribed. |  |
| PO4 Buildings, structures, infrastructure, services and utilities do not cause ground movement or vibration impacts that would cause damage or nuisance to a state-controlled transport tunnel or state-controlled transport tunnel infrastructure. | No acceptable outcome is prescribed. |  |
| PO5 Buildings, structures, infrastructure, services and utilities do not cause ground water disturbance on land for a state-controlled transport tunnel. | No acceptable outcome is prescribed. |  |
| Filling, excavation and retaining structures |  |  |
| PO6 Filling, excavation and retaining structures do not interfere with, or result in damage to, infrastructure or services in a state-controlled transport tunnel. | No acceptable outcome is prescribed. |  |
| PO7 Filling, excavation, building foundations and retaining structures do not undermine or cause subsidence of land for a state-controlled transport tunnel. | No acceptable outcome is prescribed. |  |
| PO8 Excavation, boring, piling or fill compaction during construction of a development does not result in ground movement or vibration impacts that would cause damage or nuisance to a state-controlled transport tunnel. | No acceptable outcome is prescribed. |  |
| PO9 Development does not involve blasting. | No acceptable outcome is prescribed. |  |
| PO10 Filling and excavation, building foundations and retaining structures do not cause damage to a state-controlled transport tunnel by adding or removing loading. | No acceptable outcome is prescribed. |  |
| PO11 Filling and excavation, building foundations and retaining structures do not cause ground water disturbance to a state-controlled transport tunnel corridor. | No acceptable outcome is prescribed. |  |
| PO12 Fill material from a development site does not result in contamination of a state-controlled transport tunnel corridor. | AO12.1 Fill material is free of contaminants including acid sulfate content. AND AO12.2 Compaction of fill is carried out in accordance with the requirements of AS 1289.0 2000 – Methods of testing soils for engineering purposes. |  |
| PO13 Filling and excavation in the vicinity of a state-controlled transport tunnel portal does not cause wind-blown dust nuisance in a state-controlled transport tunnel. | AO13.1 Compaction of fill is carried out in accordance with the requirements of AS 1289.0 2000 – Methods of testing soils for engineering purposes. AND AO13.2 Dust suppression measures are used during filling and excavation activities such as wind breaks or barriers and dampening of ground surfaces. |  |
| PO14 Filling and excavation material does not cause damage, obstruction or nuisance in a state-controlled transport tunnel corridor. | AO14.1 Development does not store fill, spoil or any other material in a state-controlled transport tunnel corridor. |  |
| Stormwater and drainage |  |  |
| PO15 Development does not result in an actionable nuisance or worsening of stormwater, flooding or drainage impacts in a state-controlled transport tunnel corridor or a state-controlled transport tunnel. | No acceptable outcome is prescribed. |  |
| PO16 Run-off from the development site during construction of development does not cause siltation of stormwater infrastructure affecting a state-controlled transport tunnel. | AO16.1 Run-off from the development site during construction is not discharged to stormwater infrastructure for a state-controlled transport tunnel. |  |
| PO17 Development does not cause damage to tunnel drainage structures. | No acceptable outcome is prescribed. |  |
| Access |  |  |
| PO18 Vehicular access to a development is not from a state-controlled transport tunnel. | No acceptable outcome is prescribed. |  |
| PO19 Development does not obstruct or impede existing access to a state-controlled transport tunnel. | AO19.1 Development is designed and sited to ensure existing authorised access points and access routes for maintenance and emergency works to a state-controlled transport tunnel are clear from obstructions at all times. |  |
| Network safety |  |  |
| PO20 Development involving dangerous goods adjacent to a state-controlled transport tunnel corridor does not adversely impact on the safety or operations of a state-controlled transport tunnel. | AO20.1 Development does not involve handling or storage of hazardous chemicals above the threshold quantities listed in table 5.2 of Model Planning Scheme Development Code for Hazardous Industries and Chemicals, Office of Industrial Relations, Department of Justice and Attorney-General, 2016. |  |
| Air and light |  |  |
| PO21 Development involving an accommodation activity located near a state-controlled transport tunnel portal minimises air quality impacts from a state-controlled transport tunnel. in outdoor spaces for passive recreation. | AO21.1 Each dwelling has access to an outdoor space for passive recreation which is shielded from a state-controlled transport tunnel portal by a building, solid gap-free fence, or other solid gap-free structure. |  |
| PO22 Development involving a: - childcare centre; or - educational establishment located near a state-controlled transport tunnel portal minimises air quality impacts from a state-controlled transport tunnel in outdoor education areas and outdoor play areas. | AO22.1 Each outdoor education area and outdoor play area is shielded from a state-controlled transport tunnel portal by a building, solid gap-free fence, or other solid gap-free structure. |  |
| PO23 Development involving an accommodation activity or hospital located near a state-controlled transport tunnel portal minimises lighting impacts from a state-controlled transport tunnel. | AO23.1 Buildings for an accommodation activity or hospital are designed to minimise the number of windows or transparent/translucent panels facing a state-controlled transport tunnel portal. OR AO23.2 Windows facing a state-controlled transport tunnel include treatments to block light from state-controlled transport tunnel portal. |  |

Table 5.2 Development impacting on a future state-controlled tunnel environment

| Performance outcomes | Acceptable outcomes | Response |
| --- | --- | --- |
| PO24 Development does not impede the delivery of a future state-controlled transport tunnel. | AO24.1 Development is not located on land identified as a future state-controlled transport tunnel. OR AO24.2 Development is sited and designed so that permanent buildings, structures, infrastructure, services or utilities are not located on land identified as a future state-controlled transport tunnel. OR all of the following acceptable outcomes apply: AO24.3 Structures and infrastructure located on land identified as a future state-controlled transport tunnel are able to be readily relocated or removed without materially affecting the viability or functionality of the development. AND AO24.4 Development does not involve filling and excavation of, or material changes to, land identified as a future state-controlled transport tunnel. AND AO24.5 Land is able to be reinstated to the pre-development condition at the completion of the use. | Complies with PO# / AO# Use this column to indicate whether compliance is achieved with the relevant PO or AO (or if they do not apply), and explain why |
| PO25 Filling and excavation, building foundations and retaining structures do not obstruct, undermine, or cause subsidence of land for a future state-controlled transport tunnel. | No acceptable outcome is prescribed. |  |
| PO26 Filling and excavation, building foundations and retaining structures do not cause damage to land for a future state-controlled transport tunnel by adding or removing loading. | No acceptable outcome is prescribed. |  |
| PO27 Fill material from a development site does not result in contamination of land for a future state-controlled transport tunnel. | AO27.1 Fill material is free of contaminants including acid sulfate content. AND AO27.2 Compaction of fill is carried out in accordance with the requirements of AS1289.0 2000 – Methods of testing soils for engineering purposes. |  |
| PO28 Development does not result in an actionable nuisance or worsening of stormwater, flooding or drainage impacts on land for a future state-controlled transport tunnel. | No acceptable outcome is prescribed. |  |