State code 6: Protection of state transport networks

Table 6.2 Development in general

| Performance outcomes | Acceptable outcomes | Response |
| --- | --- | --- |
| Network impacts |  |  |
| PO1 Development does not compromise the safety of users of the state-controlled road network. | No acceptable outcome is prescribed. | Complies with PO# / AO# Use this column to indicate whether compliance is achieved with the relevant PO or AO (or if they do not apply), and explain why |
| PO2 Development does not adversely impact the structural integrity or physical condition of a state-controlled road or road transport infrastructure. | No acceptable outcome is prescribed. |  |
| PO3 Development ensures no net worsening of the operating performance the state-controlled road network. | No acceptable outcome is prescribed. |  |
| PO4 Traffic movements are not directed onto a state-controlled road where they can be accommodated on the local road network. | No acceptable outcome is prescribed. |  |
| PO5 Development involving haulage exceeding 10,000 tonnes per year does not damage the pavement of a state-controlled road. | No acceptable outcome is prescribed. |  |
| PO6 Development does not require a new railway level crossing. | No acceptable outcome is prescribed. |  |
| PO7 Development does not adversely impact the operating performance of an existing railway crossing. | No acceptable outcome is prescribed. |  |
| PO8 Development does not adversely impact on the safety of an existing railway crossing. | No acceptable outcome is prescribed. |  |
| PO9 Development is designed and constructed to allow for on-site circulation to ensure vehicles do not queue in a railway crossing. | No acceptable outcome is prescribed. |  |
| PO10 Development does not create a safety hazard within the railway corridor. | No acceptable outcome is prescribed. |  |
| PO11 Development does not adversely impact the operating performance of the railway corridor. | No acceptable outcome is prescribed. |  |
| PO12 Development does not interfere with or obstruct the railway transport infrastructure or other rail infrastructure. | No acceptable outcome is prescribed. |  |
| PO13 Development does not adversely impact the structural integrity or physical condition of a railway corridor or rail transport infrastructure. | No acceptable outcome is prescribed. |  |
| Stormwater and overland flow |  |  |
| PO14 Stormwater run-off or overland flow from the development site does not create or exacerbate a safety hazard for users of a state transport corridor or state transport infrastructure. | No acceptable outcome is prescribed. |  |
| PO15 Stormwater run-off or overland flow from the development site does not result in a material worsening of operating performance of a state transport corridor or state transport infrastructure. | No acceptable outcome is prescribed. |  |
| PO16 Stormwater run-off or overland flow from the development site does not interfere with the structural integrity or physical condition of the state transport corridor or state transport infrastructure. | No acceptable outcome is prescribed. |  |
| PO17 Development associated with a state-controlled road or road transport infrastructure ensures that stormwater is lawfully discharged. | AO17.1 Development does not create any new points of discharge to a state transport corridor or state transport infrastructure. AND AO17.2 Development does not concentrate flows to a state transport corridor. AND AO17.3 Stormwater run-off is discharged to a lawful point of discharge. AND AO17.4 Development does not worsen the condition of an existing lawful point of discharge to a state transport corridor or state transport infrastructure. |  |
| Flooding |  |  |
| PO18 Development does not result in a material worsening of flooding impacts within a state transport corridor or state transport infrastructure | For a state-controlled road or road transport infrastructure, all of the following apply: AO18.1 For all flood events up to 1% annual exceedance probability, development ensures there are negligible impacts (within +/- 10mm) to existing flood levels within a state transport corridor. AND AO18.2 For all flood events up to 1% annual exceedance probability, development ensures there are negligible impacts (up to a 10% increase) to existing peak velocities within a state transport corridor. AND AO18.3 For all flood events up to 1% annual exceedance probability, development ensures there are negligible impacts (up to a 10% increase) to existing time of submergence of a state transport corridor. No acceptable outcome is prescribed for a railway corridor or rail transport infrastructure. |  |
| Drainage infrastructure |  |  |
| PO19 Drainage infrastructure does not create a safety hazard in a state transport corridor. | For a state-controlled road environment, both of the following apply: AO19.1 Drainage infrastructure associated with, or in a state-controlled road is wholly contained within the development site, except at the lawful point of discharge. AND AO19.2 Drainage infrastructure can be maintained without requiring access to a state transport corridor. For a railway environment both of the following apply: AO19.3 Drainage infrastructure associated with a railway corridor or rail transport infrastructure is wholly contained within the development site. AND AO19.4 Drainage infrastructure can be maintained without requiring access to a state transport corridor. |  |
| PO20 Drainage infrastructure associated with, or in a state-controlled road or road transport infrastructure is constructed and designed to ensure the structural integrity and physical condition of existing drainage infrastructure and the surrounding drainage network is maintained. | No acceptable outcome is prescribed. |  |
| Planned upgrades |  |  |
| PO21 Development does not impede delivery of planned upgrades of state transport infrastructure. | No acceptable outcome is prescribed. |  |

Table 6.3 Public passenger transport infrastructure and active transport

| Performance outcomes | Acceptable outcomes | Response |
| --- | --- | --- |
| PO22 Development does not damage or interfere with public passenger transport infrastructure, active transport infrastructure or public passenger services. | No acceptable outcome is prescribed. | Complies with PO# / AO# Use this column to indicate whether compliance is achieved with the relevant PO or AO (or if they do not apply), and explain why |
| PO23 Development does not compromise the safety of public passenger transport infrastructure, public passenger services and active transport infrastructure. | No acceptable outcome is prescribed. |  |
| PO24 Development does not adversely impact the operating performance of public passenger transport infrastructure, public passenger services and active transport infrastructure. | No acceptable outcome is prescribed. |  |
| PO25 Development does not adversely impact the structural integrity or physical condition of public passenger transport infrastructure and active transport infrastructure. | No acceptable outcome is prescribed. |  |
| PO26 Upgraded or new public passenger transport infrastructure and active transport infrastructure is provided to accommodate the demand for public passenger transport and active transport generated by the development. | No acceptable outcome is prescribed. |  |
| PO27 Development is designed to ensure the location of public passenger transport infrastructure prioritises and enables efficient public passenger services. | No acceptable outcome is prescribed. |  |
| PO28 Development enables the provision or extension of public passenger services, public passenger transport infrastructure and active transport infrastructure to the development and avoids creating indirect or inefficient routes for public passenger services. | No acceptable outcome is prescribed. |  |
| PO29 New or modified road networks are designed to enable development to be serviced by public passenger services. | AO29.1 Roads catering for buses are arterial or sub-arterial roads, collector or their equivalent. AND AO29.2 Roads intended to accommodate buses are designed and constructed in accordance with: - Road Planning and Design Manual, 2nd Edition, Volume 3 – Guide to Road Design; Department of Transport and Main Roads; - Supplement to Austroads Guide to Road Design (Parts 3, 4-4C and 6), Department of Transport and Main Roads; - Austroads Guide to Road Design (Parts 3, 4-4C and 6); - Austroads Design Vehicles and Turning Path Templates; - Queensland Manual of Uniform Traffic Control Devices, Part 13: Local Area Traffic Management and AS 1742.13-2009 Manual of Uniform Traffic Control Devices – Local Area Traffic Management; AND AO29.3 Traffic calming devices are not installed on roads used for buses in accordance with section 2.3.2 Bus Route Infrastructure, Public Transport Infrastructure Manual, Department of Transport and Main Roads, 2015. |  |
| PO30 Development provides safe, direct and convenient access to existing and future public passenger transport infrastructure and active transport infrastructure. | No acceptable outcome is prescribed. |  |
| PO31 On-site vehicular circulation ensures the safety of both public passenger transport services and pedestrians. | No acceptable outcome is prescribed. |  |
| PO32 Taxi facilities are provided to accommodate the demand generated by the development. | No acceptable outcome is prescribed. |  |
| PO33 Facilities are provided to accommodate the demand generated by the development for community transport services, courtesy transport services, and booked hire services other than taxis. | No acceptable outcome is prescribed. |  |
| PO34 Taxi facilities are located and designed to provide convenient, safe and equitable access for passengers. | AO34.1 A taxi facility is provided parallel to the kerb and adjacent to the main entrance. AND AO34.2 Taxi facilities are designed in accordance with: - AS2890.5–1993 Parking facilities – on-street parking and AS1428.1–2009 Design for access and mobility – general requirements for access – new building work; - AS1742.11–1999 Parking controls – manual of uniform traffic control devices - AS/NZS 2890.6–2009 Parking facilities –off street parking for people with disabilities; - Disability standards for accessible public - transport 2002 made under section 31(1) of the Disability Discrimination Act 1992; - AS/NZS 1158.3.1 – Lighting for roads and public spaces, Part 3.1: Pedestrian area (category P) lighting – Performance and design requirements; - Chapter 7 Taxi Facilities, Public Transport Infrastructure Manual, Department of Transport and Main Roads, 2015. |  |
| PO35 Educational establishments are designed to ensure the safe and efficient operation of public passenger services, pedestrian and cyclist access and active transport infrastructure. | AO35.1 Educational establishments are designed in accordance with the provisions of the Planning for Safe Transport Infrastructure at Schools, Department of Transport and Main Roads, 2011. |  |